A Z80 ASSEMBLER

BY PATRICK A CROWE

22 RINGSBURY CLOSE

PURTON SWINDON SN5 9DE ENGLAND

Introduction

This program is designed to assemble the A80 instruction set using

standard mnemonics defined in the Zilog Z80 A ssembly Language Programming

Manual.  The user is advised to obtain this book.  Rules for writing

the source code are as defined in the manual with exception that

upper case characters ONLY are recognized for labels, operators and

operands.  All defined pseudo-operations are implemented except MACRO,

ENDM, COND, and ENDC; ie no macro instructions or conditional aseembly.

All assembly time functions are implemented, plus two borrowed from Intel:

.HIGH. and .LOW.. The object code output is in absolute format only.

The assembler is written in assembly code following these rules.

- The program should be as structured as possible. Main covers the

whole assembly process, calling several subroutines, each performing

a well-defined task. These in turn call subtasks. No subroutine

is much longer than a single page. Even so, a few subroutines

grew unwieldy such as GTOD (get tokens for operands).

- A subroutine is allowed to modify only the AF register pair and

any register in which parameters are passed to it. All other

registers are preserved by pushes and pops at the beginning and

end of the routines.

In retrospect, this appears to be a useful approach except that

it inhibits use of the Z80 alternate register set. Perhaps the

rule should have been to save only register pairs BC, DE and HL.

The program occupies about 7 K bytes and can be run out of ROM.

*************************************************************************

INSTALLING THE ASSEMBLER

Six external subroutines must be provided by the user.

1. Console Input (CI): This subroutine must fetch an ASCII character from

the keyboard. The character must be returned in register A. No other

register may be affected.

2. Console Output (CO): This routine must print a character on the console.

The ASCII character will be in the C register. Only the A and C registers

may be affected.

3. List Output (LO): Save as the console output except the character must

be displayed on the list device.

4. Punch Output (PO): Same as above with output to the punch device.

5. Reader Input (RI): Same as Console Input except the source.

6. Memory Check (MEMCHK): This subroutine determines the hghest memory

that can be used for a symbol table.  The high byte of the address

must be returned in the B register, the low byte in the A register.

No other register may be affected.

I/O Linking

All I/O call are routed through the jump table at the beginning of the

assembler. The user must replace the destination addresses of these

jumps with the addresses of the six user provided subroutines.

***************************************************************************

Operating the Assembler

The assembler starts execution at hexadecimal location 100.  If the I/O

routines have been linked correctly, the assembler will give its

sign-on message.

...

(Note from David Thompson) To use this assembler, simply

enter:

CROSEASM FILENAME

where the file exists as FILENAME.Z80    The assembler will proceed with

passes 1-4.

If you have LOAD.COM on the same disk, you can enter:

LOAD FILENAME

to change the file named FILENAME.HEX to FILENAME.COM

...

Pass 1 must precede any other pass. The assembler builds the symbol table

during this pass.

Pass 2 produces the assembled listing followed by an alphabetic symbol table

listing on the list device.

Pass 3 produces the object code on the disk

Pass 4 produces an assembled listing containing those lines with errors only.

...

Notes from John Jones:

You can also enter  CROWEASM FILENAME.YY

^ Yes, do pass 3.

^ Yes, do pass 2.

Passes 1 and 4 are always run, passes 2 and 3 can be suppressed with an

"N" in the 2 or 3 pass position.

...

Error Messages

The assembled listing indicates lines containing errors by printing a

character in the first column of those lines.  Only the first error

on a line is indicated.

A: Arithmetic overflow

B: Balance error of brackets

I: Illegal character (control character in source text?)

L: Maximum line length exceeded before a semicolon.

M: Multiple devinition of sumbol

O: Stack overflow

P: Phase error. Symbol appears or changes value after pass 1.

Can be caused by forgetting pass 1.

R: Range error in relative addressing.

S: Syntax error. Covers a multitude of sins.

U: Undefined symbol

V: Illegal value

W: Reserved word used as symbol

Z: Attempt to divide by 0 at assembly time.

Pseudo-Operators

ORG nn  Sets address reference counter to the value nn.

EQU nn  Sets value of label to nn in the program. Can occur only

once for any label.

DEFL nn Sets value of a label to nn and can be repeated in the

program with different values for the same label.

END(nn) Signifies the end of the source program. If the operand nn

is present, it will be included in the ovject file as the

starting address for running the target program.

DEFB n  Defines the contents of a byte at the current reference counter

address to be n.

DEFB's' Defines the content of 1 byte of memory to be the ASCII

representation of character s.

DEFW nn Defines the contents of a 2-byte word to be nn.

DEFS nn Reserves nn bytes of memory starting at the current value

of the address reference counter.

DEFM's' Defines the content of memory to be the ASCII representation of

string s, where s must not exceed 32 characters in length.

TITLE's'Defines the program title. This title will be used in the

header of the listing. The title may contain up to 32 characters.

Assembly Time Functions

Operands may contain the aseembly time function operators hown in table 1.

Remember that expressions may not contain imbedded spaces since the space is

recognized as a delimiter.

****************************************************************************

TABLE 1

5 bit	3 bit	combined

Operator		Function		token	priority token monadic

------------------------------------------------------------------------------

+		MONADIC PLUS			0	7	07H	X

-		MONADIC MINUS			1	7	0FH	X

.NOT. or \	LOGICAL NOT			2	7	17H	X

.HIGH.		VALUE OF MOST SIGNIFICANT BYTE	3	7	1FH	X

.LOW.		VALUE OF LEAST SIGNIFICANT BYTE	4	7	27H	X

.RES.		RESULT				5	6	2EH	X

**		EXPONENTIATION			6	5	35H

*		MULTIPLICATION			7	5	3DH

/		DIVISION			8	5	45H

.MOD.		MODULO				9	5	4DH

.SHR.		LOGICAL SHIFT RIGHT		A	5	55H

.SHL.		LOGICAL SHIFT LEFT		B	5	5DH

+		ADDITION			C	4	64H

-		SUBTRACTION			D	4	6CH

.AND. or &	LOGICAL AND			E	3	73H

.OR. or ^	LOGICAL OR			F	2	7AH

.XOR.		LOGICAL EXCLUSIVE OR		10	2	82H

.EQ. or =	EQUALS				11	1	89H

.GT. or >	GREATER THAN			12 	1	91H

.LT. or <	LESS THAN			13	1	99H

.UGT.		UNSIGNED GREATER THAN		14	1	A1H

.ULT.		UNSIGNED LESS THAN		15	1	A9H

(						16	0	B0H

----------------------------------------------------------------------------

Table 1:

Assembly time function operators recognized by the Z80 assembler.

The left parens ( is included to show the token and priority

used in the expression evaluation subroutine although iit is not a function.

Priority seven is the highest.  Priorities are numbered in reverse

order from Zilog for software convenience.  This of course has no

effect on expression evaluation.  Token values are given in this list

to aid those seeking to understand the operation of the assembler.

They are irrelevant to the use of the assembler.

****************************************************************************

How it works

Eash pass uses the same processing routines except for the output

routines called by PFRLO (perform output relevant to pass number).

Operation is on a line by line basis. Ther sequence of processing for a

line is as follows:

1. Get line of source text from reader into line buffer, echoing it

if pass 2.

2. Locate label (if any) and put it in the label buffer (LABBUF).

3. Locate operator (if any) and put token for it in the operator token

buffer (ORTKBF).

4. Locate operands (if any), put tokens for them in the operand

token buffers and put related interfer values in the operand

integer buffers. (ODBT1, ODBT2, ODINT1, ODINT2).

5. Using the tokens generated in steps 3 and 4, generate object

code bytes.

6. Do output relevant to pass:

Pass 1: No output.

Pass 2: Output assembled listing.

Pass 3: Output object code.

Pass 4: Output error lines of listing.

Running a pass 1 at any time will first do complete initialization

including emptying the symbol table. The table is built during pass 1

only. During the other passes (which may be in any order) this table

is read.

The best way to understand the program is, of course, to read it, but some

information on data structures will be helpful.

Symbol Table (SYMTAB)

This table contains alphabetically arranged entries.

Entry:

- A byte specifying the number of characters in the symbol (n).

- n bytes containing ASCII symbol characters (maximum 6).

- 2 bytes containing the symbol value.

- 1 byte containing attribute bits.

bit 0: DEFL flag

bit 1: multiple definition flag

The symbol table uses up to 10 bytes per entry.  The final table entry

is followed by a null entry; a single byte containing 00. Pointer SYMEND

points to the null entry.

Symbol Buffer (SYMBUF)

The contents of the symbol buffer have the same format as an entry in

the symbol table except that for some purposes the value and attribute

bytes are not used.

Label Buffer (LABBUF)

The contents of the label buffer have te same format as an entry in

the symbol table except that the value and attribute bytes are not

included.

######################################################################################################################