David Thompson

11740 NW West Rd

Portland, Oregon 97229

503-645-3253

4/1/81 P.COM

9/7/81 PR.COM

PR.COM and P.COM are little pieces of software I wrote to do two things.

First they modify the Bios slightly to make sure that a character sent to

the display in the C register returns in the A register.  The PFM monitor

doesn't bother to do this and some programs (including adventure) require it.

Second, they put the lst: device on serial port B at 9600 baud and they

stuff two nulls (00) every time they see an ASCII LF or CR.

These characters are inserted simply to flush the SIO.

My MX 80 printer turns off the clear to send (CTS) line and

prints some text whenever it sees a carriage return or line feed.

And the two characters remaining in the SIO are ignored by the

printer as it prints the line. With this program, these lost characters

are not part of the text.  Printers that don't lose characters

simply ignore 00s so this program should work for all printers.

These programs automatically overlay the BIOS and then initiate

a system reboot.

PR.COM is for those of you whose CP/M boot includes the words

"BIG BOARD".  Your BIOS ORGS at E800.

P.COM is for those of you whose CP/M boot comes up simply

"60K CP/M version 2.2". Your BIOS ORGS at EA00.

Jumper port B (JB-5) as follows:

7 to 8  transmit data

15 to 16 request to send

19 to 20 clear to send

Prepair the MX80 cable as follows

Big Board       MX80   Big Board signal name

1               1    ground

2               3    transmit data (from Big Board)

4               6    request to send (from Big Board)

5               20   clear to send (from MX-80)

6               7    data set ready (tied high on the BB)

These programs also set the baud rate to 9600.

####################################################################################################################