DOCUMENTATION OF EXTENSIONS TO PROM BURNING SOFTWARE

EXTENSIONS BY DAVID THOMPSON

CRC calculations are done anytime the program reads a ROM.

This includes the times when it is inputting data to memory,

doing a byte/by byte compare between the ROM and the data in

memory, and when it is checking a ROM after programming. This

calculation is done on ROM data only. See below

You can also do a CRC calculation on the data in memory (RAM).

Select either 2K or 4K depending on the type of ROM you are programming.

The CRC for a block of memory lets you verify data after you've read it in

from the disk.

PROM.CRC does not support 2708s. I built my programmer to

program only 2716s and 2732s so the software didn't need to

support 2708s.

PROM.CRC also checks the entire ROM before programming to make

sure it has been erased. If any byte is not 0FFH then it reports

a ROM error and lets you choose to abort the programming or

go ahead and program the ROM. You might want to reburn a ROM

that didn't program completely the first time.

FUTURE EXTENSION:  Someone should change the byte comparison

display so that it doesn't go zipping up the screen. Either

by generating columns or by having it stop every 20 lines and

wait for a space. (Or both.)

Also, someone should set up the programmer so that no power

is applied to the programming socket until the user selects read

or program. Then the software and hardware should be able to

detect error conditions such as backward ROM or wrong part

by testing the instant power is applied.  If there is an

error the power could be removed before the part was damaged.

And Jim Monesmith says he will set it up for 2764s.

CYCLIC-REDUNDANCY-CHECK NUMBER BASED ON THE

CCITT STANDARD POLYNOMINAL:

X^16 + X^15 + X^13 + X^7 + X^4 + X^2 + X + 1

USEFUL FOR CHECKING ACCURACY OF FILE TRANSFERS.

MORE ACCURATE THAN A SIMPLE CHECKSUM.

#################################################################################################################