Big Board ZCPR notes

24 APRIL 1982

When you use sysgen to load CP/M into memory, the ccp begins

at 0980H whereas its normal location in memory is D200H for

CP/M configured by Digital Reasearch Computers of Texas or

D400H if you incorporated the BIOS yourself.

So when you use DDT to load the new CCP (which is

what ZCPR is) into memory, you must have DDT add an offset.

(in the following, you enter the items capitalized, the computer

response is lower case)

Anyway, assuming you have D200 CP/M; plus ZCPR11.HEX, SYSGEN.COM,

DDT.COM, and your present system on the disk in drive A:

Enter:

SYSGEN <CR>

source on drive    A

destination drive  <CR>

SAVE 60 CPM.COM <CR>

DDT CPM.COM <CR>

-H980,D200 <CR>  (DDT calculates offset, use D200 or D400 for your system)

db80 3780        (DDT's results, use the second number, the difference)

IZCPR11.HEX <CR> (or IZCPR.HEX if you are D400)

R3780 <CR>       (enter the second number from above as the offset)

^C               (to return to CP/M)

SYSGEN <CR>

source on drive   <CR>    (no drive name so it uses the image in memory)

destination drive  A      (to put the new system on drive A)

destination drive <CR>    (to quit)

Now try it.########################################################