CHNGPFM.ASM

CHNGPFM.COM

With the  addition  of a Z80 CTC and a 8 pin connector for

jumpers the Big  Board  has  a  real  time  clock,  but  I  was

surprised  to find that it was not supported in PFM. The reason

became very clear when I wrote the code to make it work, it  is

too  much  code  to  place  in one 2K ROM. Fortunately there is

almost a spare 2K  of  RAM  above  F000  where  one  can  place

additions  to the existing PFM and that is where I have put the

RT clock. The program called  CHNGPFM  loads  these  additional

instruction  and  links  them  to the original PFM. Having done

this, it is very easy to change other things.

The next thing I changed was the SHIFT LOCK feature  to  a

CAPS  LOCK. This requires no additions only changes, and I find

a CAPS LOCK more convenient than  a  SHIFT  LOCK  when  writing

source  code  for  assemblers  or  compilers that require upper

case.

Next I changed the code to make  the  lower  16K  bank  of

memory  more  useful. The problem here is that whenever the CRT

video RAM is accessed, the bank select bit is set, and when the

job is done the bank select  bit  is  cleared  whether  it  was

originally  cleared  or  not.  Therefore if you are executing a

program from the memory in this bank that displays something on

the CRT, when PFM completes the display you will return to bank

0 rather than bank 1. The solution is simple, save the contents

of BITDAT before the bank select bit  is  set,  and  then  upon

completion  of  the  job restore the original value. After this

change has been made you will be able to dump the  contents  of

the monitor ROM by

(1) reading the contents of BITDAT using the

PFM  I  command,

(2)  setting  bit  7 of BITDAT by using the O

command, and finally reading the ROM by using  the  D  command.

(Be  sure  you clear bit 7 in BITDAT or do a reset when you are

finished.) There are many uses you can find for this additional

RAM (see Christopher Brock's article in the August 82 issue  of

Micro  C  for  the procedure to convert this ROM space into RAM

space). An interesting use would be to load in it an  interrupt

print  routine  when you do a listing. This would enable you to

use all of your BB while the listing is produced.

-1-

Finally I changed the access to TRKTAB. I am using  a  two

sided drive as two one sided drives, and PFM uses TRKTAB (TRacK

TABle)  to  find  the  track  number  location of the disk head

whenever a new drive is selected. Since the  track  number  for

both drives A and B (and C and D) are the same because they are

the  same  physical  drive, this can create problems as well as

wasting time doing an incorrect seek. The change  that  I  made

uses  the TRKTAB entry for drive A for both drives A and B, and

the entry for drive C for both C and D. Those of you who  found

John Ballenthin's letter in the June 82 issue of Micro C a help

will find that this change is better because it is faster.

It should   be  clear  how  CHNGPFM  can  be  modified  to

incorporate any of your  changes,  the  only  trick  being  the

apparently convoluted way of doing the addressing in the code to

be added. This form is necessary  since  the  instructions  are

being  assembled  at one address and they are going to be moved

to and executed at another address.

The program TIME allows you to set  the  RT  clock  or  to

display  the time. If CHNGPFM has not been executed before TIME

is used it will display "real time  clock  is  not  installed".

Typing  "TIME ?" will display the format for setting the clock,

while typing just TIME will display the  current  time  if  the

clock  has  been  set properly. TIME also has a subroutine that

formats the date and time, and you can use this  subroutine  in

your  programs  (such  as a listing program) so that the output

can be identified with a date and time.

For those of you who want to make major PFM  changes,  the

program  LOADHEX should be of help. This program (written in C,

but a COM file is available) enables you to write a new monitor

(or modify the original source), assemble it to a HEX file, and

then overlay the original PFM with  the  new  monitor.  If  you

cannot  program  ROM's you can use the new monitor this way, or

you can debug your new monitor using this technique before  you

put  it  in  a ROM. If your assembler does not handle the Zilog

mnemonics but it will take the extended Intel mnemonics,  there

is  a translation of the PFM monitor source available that will

make it easier for you to modify and assemble a new monitor.

-2-

Include:

chngpfm.asm

time.asm

loadhex.c

rom.asm

##