PWM DRIVE MOTOR CONTROL

The pulse-width modulation is done in software.  Timing is driven

by the 1 milli-second clock interrupt.  PWM frequency is 62.5 Hz,

so the PWM on/off ratio can be zero to 100% in increments of 6%

(16 clock-ticks per pulse-width).  The PWM signal is supplied to

the output drive circuit from a parallel port bit.  When starting

the motors, several 100%-on cycles are used to get both motors

turning.  Then the appropriate duty cycle is applied. Similarly,

the duty cycle is cut to 12% several cycles before a scheduled

stopping point, providing a "soft" stop.

Each drive motor has an optical encoder disk mounted on the motor

shaft.  Since a three-pole motor (with a strong magnet) will

always come to rest in one of six possible positions, the number

of holes in the encoder disk must be a multiple of six.  Six

encoder holes would yield the maximum 100 pulse/sec rate at a

motor speed of 1000 RPM.  The encoder is positioned so that a

hole is in front of the detector when the motor is at rest.  This

allows the CPU to verify that the LED is indeed "on" when it is

enabled.  It also solves the problem of stopping on the edge of a

hole.  The encoder pulses interrupt the CPU via the Z80-CTC.

Syncronization between the two drive motors is accomplished by

trying to keep the two encoder pulses within one-half pulse-width

of each other.  Motor-1 is adjusted to keep in sync with motor-0.

Thus, if MOT-1's pulse is late, its on/off ratio is advanced 6%.

Similarly, if MOT-1's pulse is early, its on/off ratio is reduced

6%.  This limits MOT-0's range to 12-94%, since MOT-1 must have a

+/-6% adjustment factor.

When one of the pulses cause a CTC interrupt, a bit is set for

that motor and the current time is stored.  When the other

motor's pulse arrives, both motor bits are reset and the time

difference is calculated.  If the difference is more than one-

half cycle, MOT-1 is adjusted to keep pace with MOT-0.  If two

pulses in a row are received for the same motor, either the other

motor is not turning or its encoder has failed.  Trying to

increase the speed of the other motor might work, but it must

only be attempted once.  Otherwise, a motor with a bad encoder

could spin out of control.

Syncronization is independent of motor direction.  It can be done

in forward, backward, rotating (one wheel goes forward, the other

goes backward), and arc-turning mode (the wheels turn at

different speeds).  Syncronization is disabled for pivoting mode.

A syncronization factor must be specified by the caller.  This

factor is of the form "X,Y".  It specifies that for every X

pulses on MOT-0, there must be Y pulses on MOT-1.  A value of

zero for either motor effectively stops that motor while the

other motor turns (pivoting mode).  X and Y have a range of 0-10.

###################################################