FILENAME:  WHATS_UP.DOC by Don McKenzie.                Updated Sep 6, 1993

Hey, this is a new way of doing my ZLOAD Z80 Development System. If I have

goofed somewhere in the instructions, let me know.

If something isn't clear, is left un-explained, or just plain wrong, please

let me know.

And now for something completely different. A further explanation of ZLOAD

assembly procedures and other apparent and non-apparent loose ends:

The message board file MSG1P1C.Z8T hasn't been updated to ZLOAD Version 2.0

standards as yet. Perhaps this could be a practical excercise for old

message board owners, or new experimenters.

ZLOAD V2.0 Source Code Assembly guide and rules:

(1) The memory from 9F80H to 9FFFH must be reserved for ZLOAD Loader use.

(2) Line 1 must have an OFFSET value defined as 00H (ROM) or 8000H (RAM).

(3) All "ORG" statements must have an OFFSET Label.

(4) Some special code may need adjusting with special defined bytes.

(5) User stack can be set to 9FF0H over ZLOAD loader stack.

(6) First byte in code must not be 3AH. (Suggests IntelHex.)

(7) Last byte in ROM (not RAM) defined to set length at 8192 bytes.

PBUFF Source Code Example:

OFFSET  EQU     8000H            ;0H ROM, OR 8000H RAM ;(2) Offset

MSBOFF  EQU     80H              ;0H ROM, OR 80H RAM   ;(4) Special

STRPNT  EQU     9F00H            ;to 9F7FH             ;(1) Under loader

STACK   EQU     9FF0H                                  ;(5) Stack

PWRFLG  EQU     9FFFH            ;

ORG     0H+OFFSET                              ;(3) ORG

START:  NOP                      ;reserve 3 bytes      ;(6) Not 3AH

NOP                      ;for user jump

NOP

LD      SP,STACK                               ;(5) Stack

IM      1

DI

JP      START1

;PIN 16 555 TIMER

DEFS    38H+OFFSET-$                           ;(3) Same as ORG 38H

INTVEC: JP      REFRES                                 ;or 8038H but...

;fills in blanks

;PIN 17 TEST

DEFS    66H+OFFSET-$                           ;(3) Same as ORG 66H

NMIVEC: LD      BC,0FFFH                               ;or 8066H but...

CALL    DELAY                                  ;fills in blanks

ETC.

ORG     1FFFH   ;LAST BYTE IN THE 8K LIMIT     ;(7) ROM only.

DB      0FFH    ;SET ROM FILE LENGTH....       ;Comment out for Ram.

;TO 8192 BYTES

END